# - Design and Build Instructions for a Home Made Potters Wheel

Contents

- Design and Build Instructions for a Home Made Potters Wheel	1

Copyright	2

Molecular Biology Installations - ASB Software Development Limited Publications	2

FREE DOWNLOADS	2

IBM Installations - ASB Software Development Limited Publications	3

FREE DOWNLOADS	3

Contact Details	5

Bill of Materials required for the Potter’s Wheel	5

Cordless Electric Drill for driving the wheel	5

MDF 18mm thick x 500mm x 570 mm x 5 shelves	5

Plastic Oven Rack Soaking Tray	6

(eg from Lakeland)	6

Plant Saucer 30cm drip tray (used for wheel)	7

Screws, Coach Bolts and Headboard metal brackets	8

The Wheel Head	10

Foot Pedal Design	12

Hinged Wooden clamp, washer and butterfly nut	13

Hinges and Handles for Foot Pedal and carrying handles	14

Appendix  - Pictures of the Completed Potter’s Wheel	15

# Copyright

Copyright © by ASB Software Development Limited All rights reserved.

This book or any portion thereof may not be reproduced or used in any manner whatsoever without the express written permission of the publisher except for the use of brief quotations in a book review or scholarly journal.

First Printing: ISBN 978-0-244-87898-6  ,  https://asb-software.com/potters-wheels/

# Molecular Biology Installations - ASB Software Development Limited Publications

## FREE DOWNLOADS

https://doi.org/10.13140/RG.2.2.26844.33925

Installation of UCSF Chimera 1.4

https://doi.org/10.13140/RG.2.2.18753.10083

EMBOSS Analysis Of COVID-19 GENOME SEQUENCE AND INSTALLATION ON WINDOWS 10-and RHEL 8.0

# IBM Installations - ASB Software Development Limited Publications

https://bookauthority.org/books/new-ibm-watson-ebooks?t=p7zh09&s=award&book=B093ZNJY9L

https://www.amazon.co.uk/dp/B093ZNJY9L?linkCode=gs2&tag=uuid07-21

Installation and Configuration of IBM Watson Analytics and StoredIQ

## FREE DOWNLOADS

https://doi.org/10.13140/RG.2.2.34136.93449

Audit System setup on an IBM DB2 11.5 Database on RedHat Linux RHEL 8.0 using the Go Language

https://doi.org/10.13140/RG.2.2.29470.69441

Upgrade of IBM FileNet systems from DB2 10.5 FP 10 to latest DB2 11.5 version on RedHat RHEL 8.0

https://doi.org/10.13140/RG.2.2.15264.12807

Jupyter notebook and its Docker Container Installation on Linux RedHat RHEL 8.0 using VMware 15.5.2

https://doi.org/10.13140/RG.2.2.27286.50240

IBM Cognos Analytics 11.1.4 Installation - Linux Appendix for Fix for Zlib and Configuration for RHEL 8.0

https://doi.org/10.13140/RG.2.2.11682.38089

IBM Case Analyzer 5.5 Installation and Configuration On RHEL 8.0 for Case Monitor Dashboard 5.3.3 plugin

https://doi.org/10.13140/RG.2.2.35304.83208

IBM Content Search Services 5.5.3 Installation on CENTOS 8.0 and Configuration on RHEL 8.0

https://doi.org/10.13140/RG.2.2.16987.52008

Importing Case Manager solution Auto Claims example into Case Manager 5.3.3 on RHEL 8.0

https://doi.org/10.13140/RG.2.2.27057.94561

IBM CEBI Content Engine Bulk Import Document Batch Tool Example on Linux Centos 8

https://doi.org/10.13140/RG.2.2.26048.69128

Installation of LibreOffice 6.3.3.1 on Linux Centos 8.0

https://doi.org/10.13140/RG.2.2.12361.49767

IBM Case Manager Preparation of a new Target Object Store in a FileNet P8 Docker Container on CENTOS 8

https://doi.org/10.13140/RG.2.2.11327.66721

IBM Watson Explorer Foundational Components 12.0.3 Installation and Configuration on CentOS Linux 8.0

https://doi.org/10.13140/RG.2.2.12248.44803

Watson Explorer 12.0.3 oneWEX Installation on IBM Cloud CENTOS 8.0 Linux with Docker Containers

https://doi.org/10.13140/RG.2.2.20256.94729

IBM Cloud Private P8 Container CPIT Installation on CENTOS Linux 8.0

https://doi.org/10.13140/RG.2.2.36842.88007

Installation of Eclipse Codewind and Docker Compose on RHEL 8 Linux with IBM Cloud Private CE (Community Edition)
https://doi.org/10.13140/RG.2.2.20160.69129

IBM FileNet P8 Java Development on ECM Cloud Private Container P8 Examples

https://doi.org/10.13140/RG.2.2.27358.18246

IBM Cloud Private P8 Container CPIT Installation on RedHat Enterprise Linux 8.0

https://doi.org/10.13140/RG.2.2.22030.92486

Problem Resolution Procedures For fixing Software Installation Issues

https://doi.org/10.13140/RG.2.2.27345.89440

IBM BAW 18.0 Installation phase1 preprint with install of IBM Workflow Center 8.6.1.19002

https://doi.org/10.13140/RG.2.2.10491.67369

DB2 10.5 Installation on CENTOS 8.0 V1

https://doi.org/10.13140/RG.2.2.33527.57761

IBM Security Directory Services 6.4- Installation on RHEL 8.0

https://doi.org/10.13140/RG.2.2.15007.10408

WebSphere 8.5.5.15 Installation 29-06-2019 on VMWare Workstation Pro 15.1

https://doi.org/10.13140/RG.2.2.15737.83048

IBM BAW 18.0 Installation 18 07 2019 - Install of IBM Security Directory Suite 8.0.1.1 on CENTOS 8 using VMWare Workstation Pro 15.1

https://doi.org/10.13140/RG.2.2.21708.16001

Case Manager 5.3.3 Installation on RHEL 8.0 with Content Navigator 3.0.6

https://doi.org/10.13140/RG.2.2.31489.10082

Installation of Oracle 12C on the Linux operating system

https://doi.org/10.13140/RG.2.2.14590.95049

Content Navigator 3.0.6 Installation on RHEL 8.0_V1.docx

https://doi.org/10.13140/RG.2.2.21170.76480

Content Foundation 5.5.3 Installation on RHEL 8.0_V2.docx

https://doi.org/10.13140/RG.2.2.30401.51048

Case Manager Installation on RHEL 8.0_Preparation.docx

## Contact Details

For any queries on this document please contact the author’s email:

alan.bluck@asbsoftware.co.uk

## Bill of Materials required for the Potter’s Wheel

### Cordless Electric Drill for driving the wheel

Eg Parkside Cordless Drill PABS 20-Li D4 With 2ah Battery Charger Auto Spindle

The important point above is that there must be a screw-in handle on the drill which can be inserted on either side with the hole allowing a 6mm threaded bolt to be screwed through to hold the drill firmly against a metal back plate.

NB Beware of the drills advertised as a bare unit!:

Sometimes, although the picture shows a battery attached, it may be sold as a Bare Unit with no 20 Volt Lithium Battery supplied!

NB Also in my pictures below I used the older Parkside Drill Model PSBS 24 A1 (which I used, but is no longer sold)

With a 24Volt Lithium battery I used this since it was idle in a box in the garage.

(It might be possible to get one of these second-hand!)

### MDF 18mm thick x 500mm x 570 mm x 5 shelves

(From search online for a Kitchen Base Unit with a white matt finish )

NB I first used 18mm x 460mm x 430mm MDF shelving  (and this works fine, as a minimum size) as I had these left over in my garage from the kitchen we had installed..

(I have now ordered 5 base kitchen units)

I intend to build another 4 wheels from these 5 units.

Above are the MDF boards propped in the positions they will be placed in plus the wheel head components.

(The plywood is 6mm Marine ply to be cut into square bats for the wheel)

### Plastic Oven Rack Soaking Tray

### (eg from Lakeland)

Ref

https://www.lakeland.co.uk/20228/Oven-Rack-Soaking-Tray

Internal – 55 x 40cm x 12cm high (Product number: 20228)

Used as shown for the wheel ‘bowl’ (NB the wheel above is green, but I have selected terracotta in the example below.)

### Plant Saucer 30cm drip tray (used for wheel)

### Screws, Coach Bolts and Headboard metal brackets

- The coat hanger metal is used to create a spring by wrapping it around a 6mm Iron bar:

- This spring is used as shown above to counter-load the bolt which presses down as a lever on the drill switch:

c) This is the bolt on the drill side of the central supporting panel.

d) There is a net-curtain, plastic coated steel flexible wire wrapped as shown around the bolt to pull the drill switch down.

### The Wheel Head

Screws are 50mm cross-head plasterboard dry-wall screws:

Use water /gas piping end or Search for Water Butt spare parts

25mm (O.D.) Hose Tail

This can be used as an alternative to the brass piping shown above.

### Foot Pedal Design

Hinge end                                          Round Hook end

Curtain wire and circular hooks were used to attach the foot pedal to the bolt fixed across the drill switch:

### Hinged Wooden clamp, washer and butterfly nut

Open clamp                                                                                 Closed clamp with washer and butterfly nut

The washer and butterfly nut enlarged.

### Hinges and Handles for Foot Pedal and carrying handles

## Appendix  - Pictures of the Completed Potter’s Wheel